The validation of the error equivalent method

In order to validate the rationality and effectiveness of the error equivalent method proposed in Section 2, simulations on regular and complex elastic structures are conducted in this document.

1. Core idea of the error equivalent method

As shown in Fig. (2) in the paper, the final configuration of end frame for complicated structure considering geometric error and flexible deformation can be treated as a transformation matrix from the nominal one.

where  is the deformation twist of the flexible link.  and  are the initial geometric twist and geometric error twist for the link.

For a single structure, the geometric error is independent of the flexible deformation. Consequently, the end coordinate transformations are independent when irrespective of the presence of geometric errors in the component. When the flexible deformation is ignored, the kinematic equation can be transferred as

where the geometric error twist can be represented in the group of Lie algebra.

Further, the deformation can be represented according to the mechanic theory and screw theory, just shown in [1].

where  is the compliance matrix of the flexible structure.  is the force wrench applied to the end’s frame.

Meanwhile, the deformation twist can be obtained through matrix transformation

Therefore, the final transformation matrix of the flexible link can be obtained as

2. Simulations on flexible link with regular and irregular section

2.1 Validate on a regular section

For those flexible links with regular sections, the flexible deformation can be obtained by the analytic equation in Mechanics [2-3]. In this subsection, a flexible link with a circle section is adopted to verify the effectiveness based on the analytical method, just as shown in Fig. (A1).

Figure A1: A spatial flexible rod with length L

For the flexible rod, the length L is the actual one, while the nominal length is Ln. The length δL is assigned as geometric errors for this link. Consequently, the relationship between the nominal one and the actual one is L = Ln+δL. Meanwhile, the compliance matrix of this flexible rod is

where A represents the area of cross section, Ixx, Iyy, and Izz are the three principal area moments, while E and G are the elastic and shear modulus, respectively.

Consequently, two types of simulations will be used to verify the method's effectiveness. Firstly, forces with different directions are exerted to the end frame under the same incremental increase, and the position deviation between before and after deflection can be obtained based on Equation 1. Secondly, the force is gradually incrementally increased with different load changes along a single direction. The results can be presented in Figure A2. From Figure A2(a), the position deviation is linear with respect to the loads under different directions. The different slopes are mainly due to the different stiffness characteristics. In particular, the stiffness properties along the x and y directions are the same, while the stiffness along the z direction is maximal. All results are identical to the stiffness properties of the elastic structure. Similarly, according to Figure A2(b), the relationship between the position deviation along the x direction and the forces is also linear. All those results can confirm the correctness of the error equivalent method.

| (a) Position deviations with different directions | (b) Position deviations with different gaining loads |
| --- | --- |
| Figure A2: Position deviations compare with undeformed frame of end-effector under gradually increasing load |  |

2.2 Validation on irregular section

Further, simulation on a complicated structure is conducted to verify the effectiveness of the error equivalent method. The main part of the KR5 industrial robot is adopted in this part. Firstly, the geometric error is artificially imposed on the 3D model, just as shown in Figure A3. Then, 36 forces (6 groups of forces are carefully selected, and the rest are randomly obtained) are applied sequentially to the end of the component to obtain the corresponding deformations based on Abaqus. The carefully selected six groups of force and deformation are used to solve the compliance matrix of the component, and the rest are used for verification.

Figure A3: The deformation of the irregular component

Then, the compliance matrix can be obtained as

where  and  are the ith applied wrench and  the corresponding twist deformation that satisfies the relation . Here,  is the number of the preloaded wrenches, and represents the Moore–Penrose pseudoinverse of the 6×m matrix constructed by the force twists.

It should be pointed out that all the wrenches and twists in Eq. (7) must be represented with respect to the same reference frame. Then, the numerical result of the compliance matrix C from the FEA-based simulation can be calculated in terms of the force and deflection datum, as illustrated in Table 1, collected from any commercial CAE code (here, we used Abaqus/Standard 6.12). To eliminate the antisymmetric part of the stiffness matrix due to the finite external loads, a symmetrization operation is performed to the original one, which yields to the link’s final structural compliance , as shown in Eq. (8).

| Applied wrenches (N·mm, N) | Deformation twists (rad, mm) |
| --- | --- |

The position and orientation errors between the analytical equation (1) and the FEA ones can be found in Figure A4.

| (a) Residual position error | (b) Residual orientation error |
| --- | --- |
| Figure A4: Residual errors between the analytical results and FEA results on verification configurations |  |

From the results shown in Figure 5, both position errors and orientation errors are small enough that the coordinate transformation of the end based on the error equivalent method is the same as the FEA method.

3. Conclusion

In this document, detailed derivation and simulations are proposed to verify the effectiveness of the error equivalent method. The derivation can illustrate the effect of geometric error and deformation on the end. And simulations on regular and irregular flexible parts are also conducted separately to demonstrate the method's effectiveness. The analytical equation is adopted to the regular part to show the actual position deviations along the same forces. Meanwhile, the comparisons between the error equivalent method and the FEA method are also obtained for the irregular part. The results show that both position and orientation errors are small to verify the correctness of the error equivalent method.

Reference:

Selig J M, Ding X. A screw theory of static beams[C]//Proceedings 2001 IEEE/RSJ International Conference on Intelligent Robots and Systems. Expanding the Societal Role of Robotics in the the Next Millennium (Cat. No. 01CH37180). IEEE, 2001, 1: 312-317.

Chen G, Zhang Z, Wang H. A general approach to the large deflection problems of spatial flexible rods using principal axes decomposition of compliance matrices[J]. Journal of Mechanisms and Robotics, 2018, 10(3): 031012.

Fenner R T, Reddy J N. Mechanics of solids[M]. CRC Press, 1991.